CSA 0706   COMPUTER NETWORKS    SLOT C

Q1. 1. A company needs to transfer a 150 MB design file from a client computer to

a remote server over a network with a bandwidth of 50 Mbps. The Transport

Layer divides the file into 1500-byte segments, while the Data Link Layer

encapsulates each segment into frames by adding 40 bytes of header and

trailer. Calculate the total number of segments and frames transmitted, the total

transmission overhead introduced by the Data Link Layer, and estimate the total

transmission time (ignoring propagation delay). Explain how the Transport and

Data Link layers ensure reliable communication

File Transfer Across a Network

Given:

- File size = 150 MB = 150,000,000 bytes

- Segment size = 1500 bytes

- Data Link overhead = 40 bytes/frame

- Bandwidth = 50 Mbps

Calculations:

- Total segments = 150,000,000 / 1500 = 100,000 segments

- Total frames = 100,000 frames (1 frame per segment)

- Total D-L overhead = 100,000 × 40 = 4,000,000 bytes (4 MB)

- Total data transmitted = 150,000,000 + 4,000,000 = 154,000,000 bytes = 1,232,000,000 bits

- Transmission time = 1,232,000,000 / 50,000,000 = 24.64 seconds

Explanation:
The Transport Layer ensures reliability by segmenting data, numbering segments sequentially, using acknowledgments (ACKs), retransmitting lost segments, and applying flow/congestion control (e.g., TCP). The Data Link Layer ensures reliability over each physical hop by framing data, adding error-detection codes (CRC/checksum) in the trailer, and performing error control and node-to-node flow control (e.g., via ARQ).

Q2. A Transport Layer protocol uses a sliding window size of 6 frames. Each

frame carries 1024 bytes of user data. If the sender transmits 48 frames,

calculate the number of transmission windows required. If one frame in every

window is lost and must be retransmitted, determine the total number of

retransmissions. Explain how flow control improves communication efficiency.

Sliding Window Flow Control

Given:

- Window size = 6 frames

- Frame data size = 1024 bytes

- Total frames sent = 48

- 1 frame lost per window

Calculations:

- Number of windows = 48 / 6 = 8 windows

- Retransmissions = 1 lost frame × 8 windows = 8 retransmissions

Explanation:
Flow control (sliding window) lets the sender transmit multiple frames before waiting for an acknowledgment, instead of stopping after every single frame. This improves efficiency by keeping the channel busy, reduces idle time, and still lets the receiver control the pace of data so it isn't overwhelmed — balancing speed with reliability.

Q3. 3. A 12,000-byte IP packet is transmitted through a router where the Maximum

Transmission Unit (MTU) is 1500 bytes. Assuming each fragment requires a

20-byte IP header, calculate the total number of fragments created and the total

header overhead introduced. Explain how the Network Layer ensures successful

delivery of fragmented packets to the destination.

IP Packet Fragmentation

Given:

- Packet size = 12,000 bytes

- MTU = 1500 bytes

- IP header per fragment = 20 bytes

Calculations:

- Payload per fragment = 1500 − 20 = 1480 bytes

- Number of fragments = ⌈12,000 / 1480⌉ = 9 fragments

- Total header overhead = 9 × 20 = 180 bytes

Explanation:
The Network Layer ensures successful delivery of fragments using the packet identification field (to group fragments of the same original packet), the fragment offset field (to reorder them), and the More Fragments (MF) flag (to signal the last fragment). The destination host reassembles all fragments using these fields before passing data up to the Transport Layer.

Q4.  A Transport Layer protocol uses a sliding window size of 6 frames. Each

frame carries 1024 bytes of user data. If the sender transmits 48 frames,

calculate the number of transmission windows required. If one frame in every

window is lost and must be retransmitted, determine the total number of

retransmissions. Explain how flow control improves communication efficiency.

Sliding Window Flow Control

Given:

- Window size = 6 frames

- Frame data size = 1024 bytes

- Total frames sent = 48

- 1 frame lost per window

Calculations:

- Number of windows = 48 / 6 = 8 windows

- Retransmissions = 1 lost frame × 8 windows = 8 retransmissions

Explanation:
Flow control (sliding window) lets the sender transmit multiple frames before waiting for an acknowledgment, instead of stopping after every single frame. This improves efficiency by keeping the channel busy, reduces idle time, and still lets the receiver control the pace of data so it isn't overwhelmed — balancing speed with reliability.

Q5. A multimedia application transfers a 600 MB video file. The Session Layer

inserts a synchronization checkpoint after every 60 MB of transmitted data.

During transmission, a failure occurs after 390 MB has been transferred.

Determine the number of checkpoints created before failure and calculate the

amount of data that must be retransmitted after recovery. Explain the importance

of synchronization in the Session Layer.

Session Layer Synchronization Checkpoints

Given:

- Total file size = 600 MB

- Checkpoint every 60 MB

- Failure occurs after 390 MB transferred

Calculations:

- Checkpoints created (at 60, 120, 180, 240, 300, 360 MB) = 6 checkpoints

- Last checkpoint before failure = 360 MB

- Data to retransmit = 390 − 360 = 30 MB

Explanation:
The Session Layer manages dialog control and synchronization between systems. By inserting checkpoints, it allows a failed transfer to resume from the last checkpoint rather than restarting entirely, saving significant time and bandwidth — critical for large file/session recovery.

Q6.Before transmission, the Presentation Layer compresses a 900 MB file by

40%. The compressed data is encrypted, adding 5% overhead to the

compressed size. Calculate the compressed file size, the encrypted file size, and

the total percentage reduction compared to the original file. Explain how the

Presentation Layer improves transmission efficiency and security.

Presentation Layer — Compression & Encryption

Given:

- Original file size = 900 MB

- Compression reduces size by 40%

- Encryption adds 5% overhead to compressed size

Calculations:

- Compressed size = 900 × (1 − 0.40) = 540 MB

- Encrypted size = 540 × 1.05 = 567 MB

- Total reduction = (900 − 567) / 900 × 100 = 37% reduction

Explanation:
The Presentation Layer improves efficiency by compressing data (reducing size and transmission time/bandwidth usage) and improves security by encrypting data (protecting confidentiality and integrity), while also handling data format translation between systems.

Q7. An FTP application transfers 3 GB of data over a network with an effective

throughput of 120 Mbps. The Application Layer divides the data into requests

of 3 MB each. Calculate the total number of requests generated and estimate the

total transmission time in seconds. Discuss how the Application Layer provides

reliable services to users.

FTP Application Layer Transfer

Given:

- Total data = 3 GB = 3,000,000,000 bytes

- Request size = 3 MB = 3,000,000 bytes

- Effective throughput = 120 Mbps

Calculations:

- Number of requests = 3,000,000,000 / 3,000,000 = 1,000 requests

- Total bits = 3,000,000,000 × 8 = 24,000,000,000 bits

- Transmission time = 24,000,000,000 / 120,000,000 = 200 seconds

Explanation:
The Application Layer (FTP) provides reliable service to users by relying on the underlying Transport Layer (TCP) for guaranteed, in-order delivery, while itself managing user authentication, session/connection setup, and structured request-response exchanges for file transfer.

Q8. A packet experiences the following processing delays at different OSI layers:

Application Layer = 4 ms, Transport Layer = 3 ms, Network Layer = 5 ms, Data

Link Layer = 2 ms, and Physical Layer = 1 ms. The propagation delay is 18 ms,

and the transmission delay is 12 ms. Calculate the total end-to-end delay

experienced by the packet and explain how each layer contributes to the

communication process.

End-to-End Delay Across OSI Layers

Given:

- Application = 4 ms, Transport = 3 ms, Network = 5 ms, Data Link = 2 ms, Physical = 1 ms

- Propagation delay = 18 ms

- Transmission delay = 12 ms

Calculations:

- Total processing delay = 4 + 3 + 5 + 2 + 1 = 15 ms

- Total end-to-end delay = 15 + 18 + 12 = 45 ms

Explanation:
Each OSI layer adds its own processing delay as data passes through it (encapsulation/decapsulation, error checking, routing decisions, etc.). Propagation delay depends on physical distance/medium, and transmission delay depends on link bandwidth — together with per-layer processing delays, they make up the total end-to-end delay experienced by a packet.

Q9.A network transfers 80 MB of useful data using frames of 1600 bytes, where

each frame contains 80 bytes of protocol overhead added by different OSI

layers. Calculate the total number of frames required, the total overhead

transmitted, and the transmission efficiency as a percentage. Explain how

protocol overhead affects network performance.

Frame Overhead & Transmission Efficiency

Given:

- Useful data = 80 MB = 80,000,000 bytes

- Frame size = 1600 bytes

- Overhead per frame = 80 bytes

Calculations:

- Payload per frame = 1600 − 80 = 1520 bytes

- Number of frames = ⌈80,000,000 / 1520⌉ = 52,632 frames

- Total overhead = 52,632 × 80 = 4,210,560 bytes

- Total data transmitted = 52,632 × 1600 = 84,211,200 bytes

- Efficiency = (1520 / 1600) × 100 = 95%

Explanation:
Protocol overhead (headers/trailers added by various layers) reduces the proportion of a frame carrying actual useful data. Higher overhead relative to payload lowers transmission efficiency, meaning more bandwidth and time are consumed for the same amount of useful data — this is why larger payload-to-overhead ratios are preferred in network design.

Q10. A hospital transfers a patient&#39;s medical record of 240 MB through an IoT-

enabled healthcare network. The Presentation Layer compresses the data by

30%, the Transport Layer divides the compressed data into 1400-byte

segments, and the Data Link Layer adds 60 bytes of overhead to each frame.

Calculate the compressed file size, the total number of segments transmitted, the

total protocol overhead introduced, and the final amount of data transmitted

across the Physical Layer. Explain how multiple OSI layers work together to

ensure secure and reliable communication.

Healthcare Data Transfer (IoT Network)

Given:

- Original record size = 240 MB

- Presentation Layer compression = 30%

- Transport Layer segment size = 1400 bytes

- Data Link Layer overhead = 60 bytes per frame

Calculations:

- Compressed size = 240 × (1 − 0.30) = 168 MB = 168,000,000 bytes

- Number of segments = 168,000,000 / 1400 = 120,000 segments

- Total Data Link overhead = 120,000 × 60 = 7,200,000 bytes

- Final data across Physical Layer = 168,000,000 + 7,200,000 = 175,200,000 bytes (175.2 MB)

Explanation:
Multiple OSI layers work together to ensure secure, reliable delivery: the Presentation Layer compresses (and typically encrypts) sensitive medical data; the Transport Layer segments the data and guarantees reliable, ordered delivery with error/flow control; the Network Layer routes segments across the IoT network; and the Data Link Layer frames each segment with error-detection overhead for reliable transmission over each physical link — together enabling secure, efficient healthcare data communication.

.