SIMATS ENGINEERING

ASSESSMENT TOOL 1

COURSE CODE:  CSA0713

COURSE NAME: COMPUTER NETWORKS

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 40%

Annexure A

ANALYTICAL PROBLEM SOLVING

Code of Conduct:

I ____KAVINILAVU.S(192565038)_____ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

Kavinilavu.S

Annexure A

ANALYTICAL PROBLEM SOLVING

1. Suppose we want to transmit the message 11100011 and protect it from errors using the CRC polynomial X3+1.

i.) Use polynomial long division to determine the message that should be transmitted.

ii) Suppose the leftmost bit of the message is inverted due to noise on the transmission link. What is the result of the receiver's CRC calculation? How does the receiver know that an error has occurred?

SOLUTION:

Generator polynomial: X³ + 1 → binary 1001 (degree 3 → append 3 zero bits to the message before dividing)

i) Polynomial long division:

Message = 11100011 Message + 3 appended zeros = 11100011000 (11 bits, call them b1...b11 = 1,1,1,0,0,0,1,1,0,0,0)

Mod-2 (XOR) division by 1001, one bit brought down at a time:

| Step | Register (4 bits) | Leading bit | Action | Result after XOR | Drop leading bit, bring down next dividend bit |
| --- | --- | --- | --- | --- | --- |
| 1 | 1110 (b1-b4) | 1 | XOR with 1001 | 0111 | → 111 + b5(0) = 1110 |
| 2 | 1110 | 1 | XOR with 1001 | 0111 | → 111 + b6(0) = 1110 |
| 3 | 1110 | 1 | XOR with 1001 | 0111 | → 111 + b7(1) = 1111 |
| 4 | 1111 | 1 | XOR with 1001 | 0110 | → 110 + b8(1) = 1101 |
| 5 | 1101 | 1 | XOR with 1001 | 0100 | → 100 + b9(0) = 1000 |
| 6 | 1000 | 1 | XOR with 1001 | 0001 | → 001 + b10(0) = 0010 |
| 7 | 0010 | 0 | no XOR (leading bit 0) | 0010 | → 010 + b11(0) = 0100 |
| 8 | 0100 | 0 | no XOR (leading bit 0) | 0100 | → 100 (no more bits — this is the remainder) |

Remainder (CRC checksum) = 100

Transmitted message = original message + CRC = 11100011 + 100 = 11100011100

Quotient bits collected along the way (not needed for the answer, just for completeness): 1 1 1 1 1 1 0 0.

Verification (polynomial algebra, cross-check): M(x) = x¹⁰+x⁹+x⁸+x⁴+x³, G(x) = x³+1. Dividing degree by degree in GF(2) reduces step by step to a final remainder of x² = binary 100 — matching the long-division table above.

ii) Leftmost bit inverted during transmission

Transmitted frame: 1 1100011100 Leftmost bit flips (1→0). Received frame: 01100011100

The receiver repeats the same division (by 1001) on the received 11-bit frame:

| Step | Register | Leading bit | Action | Result | Drop leading bit, bring down next bit |
| --- | --- | --- | --- | --- | --- |
| 1 | 0110 (b1-b4) | 0 | no XOR | 0110 | → 110 + b5(0) = 1100 |
| 2 | 1100 | 1 | XOR 1001 | 0101 | → 101 + b6(0) = 1010 |
| 3 | 1010 | 1 | XOR 1001 | 0011 | → 011 + b7(1) = 0111 |
| 4 | 0111 | 0 | no XOR | 0111 | → 111 + b8(1) = 1111 |
| 5 | 1111 | 1 | XOR 1001 | 0110 | → 110 + b9(1) = 1101 |
| 6 | 1101 | 1 | XOR 1001 | 0100 | → 100 + b10(0) = 1000 |
| 7 | 1000 | 1 | XOR 1001 | 0001 | → 001 + b11(0) = 0010 |
| 8 | 0010 | 0 | no XOR | 0010 | → 010 (no more bits — final remainder) |

Receiver's CRC calculation → remainder = 010

Shortcut cross-check: flipping only the leftmost bit is equivalent to XOR-ing the transmitted frame with x¹⁰. Since the transmitted frame divides G(x) exactly (remainder 0), the corrupted frame's remainder equals x¹⁰ mod G(x). Because G(x)=x³+1 ⟹ x³≡1, we get x¹⁰=(x³)³·x ≡ 1·x = x = 010.

How the receiver detects the error: A correctly received frame is always an exact multiple of the generator polynomial, so dividing it by the generator must leave remainder 0. Here the computed remainder is 010 (decimal 2), which is non-zero — this mismatch is the receiver's signal that one or more bits were corrupted in transit, so it discards the frame and requests retransmission (or flags it as an error), even though it doesn't know exactly which bit was wrong (CRC detects errors, it doesn't locate/correct them).

2. a) A network with bandwidth of 10 Mbps can pass only an average of 12,000 frames per minute with each frame carrying an average of 10,000 bits. What is the throughput of this network?

b) A network has a bandwidth of 3000 Hz (300 to 3300 Hz) assigned for data communications. The signal- to-noise ratio is 3162. Find the channel capacity

SOLUTION:

a) Throughput

Given: Bandwidth = 10 Mbps, average frames = 12,000 per minute, average frame size = 10,000 bits.

Step 1 — convert frame rate to per second: 12,000 frames/min ÷ 60 s/min = 200 frames/second

Step 2 — throughput = frame rate × bits per frame: Throughput = 200 frames/s × 10,000 bits/frame = 2,000,000 bits/s

Throughput = 2,000,000 bps = 2 Mbps

Interpretation: Even though the link's bandwidth (raw capacity) is 10 Mbps, the actual measured throughput is only 2 Mbps — the network is using just 20% of its available capacity, with the rest lost to overhead, idle time, or other inefficiencies.

b) Channel capacity (Shannon's theorem)

Given: Bandwidth B = 3000 Hz (the 300–3300 Hz voice-grade band), SNR = 3162 (a ratio, not dB).

Formula: C = B × log₂(1 + SNR)

Step 1: 1 + SNR = 1 + 3162 = 3163

Step 2 — compute log₂(3163):

log₂(3163) = log₁₀(3163) / log₁₀(2) log₁₀(3163) ≈ 3.5001 (because 3163 ≈ 3.162 × 10³,

=> 10^0.5 = 3.1623, so log₁₀(3.162) ≈ 0.5) =>log₁₀(2) = 0.30103

=> log₂(3163) = 3.5001 / 0.30103 ≈ 11.63

Step 3 — multiply by bandwidth: C = 3000 × 11.63 ≈ 34,884 bps ≈ 34.88 kbps

Interpretation: This is the theoretical maximum error-free bit rate this 3000 Hz voice-grade channel can carry at the given signal-to-noise ratio — any encoding scheme faster than this will necessarily introduce errors.

3. A multispecialty hospital uses wireless patient-monitoring devices to continuously transmit patients' heart rate and blood pressure readings to a central monitoring station. Since the data is transmitted over a wireless channel, electromagnetic interference occasionally changes one bit during transmission. To ensure that doctors receive accurate patient information without waiting for retransmission, the hospital implements the Hamming (12,8) error-correcting code.

SOLUTION:

Why Hamming (12,8) fits this use case: for m=8 data bits, the number of parity bits r must satisfy 2ʳ ≥ m+r+1.

r=4 works (2⁴=16 ≥ 13), giving a 12-bit codeword capable of correcting any single-bit error instantly at the receiver, with no retransmission round-trip — exactly what's needed for continuous, time-critical vitals data.

Step 1 — Lay out the 12 bit positions. Parity bits occupy the power-of-2 positions (1, 2, 4, 8); the 8 data bits fill the remaining positions (3, 5, 6, 7, 9, 10, 11, 12):

| Position | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Role | P1 | P2 | D1 | P4 | D2 | D3 | D4 | P8 | D5 | D6 | D7 | D8 |
| Data bit (11100011) | – | – | 1 | – | 1 | 1 | 0 | – | 0 | 0 | 1 | 1 |

Step 2 — Compute each parity bit using even parity (each parity bit is set so the XOR of all bits it covers, including itself, equals 0):

- P1 covers positions 1,3,5,7,9,11 → data bits at those positions: D1=1, D2=1, D4=0, D5=0, D7=1 XOR: 1⊕1⊕0⊕0⊕1 = 1 → P1 = 1

- P2 covers positions 2,3,6,7,10,11 → D1=1, D3=1, D4=0, D6=0, D7=1 XOR: 1⊕1⊕0⊕0⊕1 = 1 → P2 = 1

- P4 covers positions 4,5,6,7,12 → D2=1, D3=1, D4=0, D8=1 XOR: 1⊕1⊕0⊕1 = 1 → P4 = 1

- P8 covers positions 8,9,10,11,12 → D5=0, D6=0, D7=1, D8=1 XOR: 0⊕0⊕1⊕1 = 0 → P8 = 0

Step 3 — Final 12-bit codeword (positions 1–12):

1 1 1 1 1 1 0 0 0 0 1 1

Step 4 — Single-bit error correction in action. Suppose electromagnetic interference flips the bit at position 7 (0→1) during wireless transmission.

Received codeword: 1 1 1 1 1 1 1 0 0 0 1 1 (position 7 now reads 1 instead of 0)

The receiver recomputes the same four parity checks on the received bits:

| Check | Covers | Received values | XOR result | Match? |
| --- | --- | --- | --- | --- |
| C1 | 1,3,5,7,9,11 | 1,1,1,1,0,1 | 1 | mismatch |
| C2 | 2,3,6,7,10,11 | 1,1,1,1,0,1 | 1 | mismatch |
| C4 | 4,5,6,7,12 | 1,1,1,1,1 | 1 | mismatch |
| C8 | 8,9,10,11,12 | 0,0,0,1,1 | 0 | match |

Syndrome (error position) = C8 C4 C2 C1 = 0 1 1 1₂ = decimal 7

This exactly identifies position 7 as the corrupted bit. The receiver flips it back (1→0), fully recovering the original vitals reading 111111000011 without any retransmission — the key operational advantage that makes Hamming codes suitable for real-time medical telemetry, where waiting for a retransmit could mean stale or missing readings.

4. a) A company uses the IP address 172.16.10.0/24 and has implemented subnetting using a /27 subnet mask for its internal departments. Using the given subnet mask, apply subnetting techniques to calculate the total number of subnets created and the number of usable hosts per subnet. Determine the subnet to which the IP address 172.16.10.78 belongs. Identify the network address and broadcast address for that subnet. Also, determine whether the IP address 172.16.10.95 falls within the same subnet range based on the given configuration.

b) A network uses the Internet Protocol version 6 address 2001:0db8:0000:0000:0000:ff00:0042:8329. Apply IPv6 addressing rules to compress the given address into its shortest form and then expand the compressed address back to its full notation. Using a prefix length of /64, determine the network prefix of the given address. Also, calculate the total number of possible host addresses available within this network based on the given prefix length.

SOLUTION:

Step 1 — Write the original network address in full 32-bit binary.

172.16.10.0 splits into four octets: 172. 16. 10. 0

| Octet | Decimal | Binary |
| --- | --- | --- |
| 1st | 172 | 10101100 |
| 2nd | 16 | 00010000 |
| 3rd | 10 | 00001010 |
| 4th | 0 | 00000000 |

Full 32-bit address: 10101100.00010000.00001010.00000000

Step 2 — Write the /27 mask in full 32-bit binary. A /27 mask means the first 27 bits are 1s and the remaining 5 bits are 0s:

11111111.11111111.11111111.11100000

Converting the last octet 11100000 to decimal: 128+64+32 = 224 So the /27 mask in dotted-decimal is 255.255.255.224

Step 3 — Identify how many bits were borrowed for subnetting.

Original prefix = /24 (first 3 octets fixed = 11111111.11111111.11111111).

New prefix = /27. Borrowed bits = 27 − 24 = 3 bits (these are the extra 1s that spilled into the 4th octet: 111 in 11100000)

Step 4 — Number of subnets created. Each borrowed bit doubles the number of possible subnets:

2³ = 2 × 2 × 2 = 8 subnets

Step 5 — Host bits remaining, and usable hosts per subnet. Total address bits = 32. Bits used for network+subnet = 27. Bits left for hosts = 32 − 27 = 5 host bits. Total addresses per subnet = 2⁵ = 32. Usable hosts = total addresses − (1 network address − reserved, 1 broadcast address − reserved): 2⁵ − 2 = 32 − 2 = 30 usable hosts per subnet

Step 6 — Block size (the "jump" between consecutive subnets). Block size = 256 − (last octet of the mask) = 256 − 224 = 32 This means each subnet's 4th octet starts at a multiple of 32: 0, 32, 64, 96, 128, 160, 192, 224.

Step 7 — Build the subnet table by stepping through the block size. Starting from 172.16.10.0 and adding 32 each time to get the next network address; the broadcast address of each block is always (next network address − 1):

| Subnet # | 4th-octet range | Network address | Usable host range | Broadcast address |
| --- | --- | --- | --- | --- |
| 1 | 0–31 | 172.16.10.0 | .1 – .30 | 172.16.10.31 |
| 2 | 32–63 | 172.16.10.32 | .33 – .62 | 172.16.10.63 |
| 3 | 64–95 | 172.16.10.64 | .65 – .94 | 172.16.10.95 |
| 4 | 96–127 | 172.16.10.96 | .97 – .126 | 172.16.10.127 |
| 5 | 128–159 | 172.16.10.128 | .129 – .158 | 172.16.10.159 |
| 6 | 160–191 | 172.16.10.160 | .161 – .190 | 172.16.10.191 |
| 7 | 192–223 | 172.16.10.192 | .193 – .222 | 172.16.10.223 |
| 8 | 224–255 | 172.16.10.224 | .225 – .254 | 172.16.10.255 |

Step 8 — Locate 172.16.10.78 by binary ANDing with the mask. Only the 4th octet needs converting since the first three octets are identical to the networks:

78 in binary: 64+8+4+2 = 01001110 Mask's 4th octet: 11100000

01001110 (78)

AND

11100000 (mask)

-----------

01000000   = 64

The AND result's 4th octet = 64, so:

→ 172.16.10.78 belongs to Subnet 3: 172.16.10.64/27

- Network address: 172.16.10.64

- Broadcast address: 172.16.10.95 (next subnet's network address 172.16.10.96, minus 1)

- Usable host range: 172.16.10.65 – 172.16.10.94

Step 9 — Check whether 172.16.10.95 falls in the same subnet. 95 in binary: 64+16+8+4+2+1 = 01011111

01011111   (95)

AND

11100000   (mask)

-----------

01000000   = 64

The AND result is also 64 — the same subnet block as .78. So yes, 172.16.10.95 falls within Subnet 3's range (172.16.10.64 – 172.16.10.95).

However, look at the host portion of .95: the last 5 bits are 11111 — all 1s. In any subnet, the address whose host bits are all 1s is by definition the broadcast address, not an assignable host address. So 172.16.10.95 is inside the subnet's block, but it's reserved for broadcast, not for a device.

### b) IPv6 address: 2001:0db8:0000:0000:0000:ff00:0042:8329

An IPv6 address has 8 groups of 16 bits each (4 hex digits per group), separated by colons — 8 × 16 = 128 bits total.

Step 1 — List all 8 groups as given:

| Group | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Value | 2001 | 0db8 | 0000 | 0000 | 0000 | ff00 | 0042 | 8329 |

Step 2 — Compression rule 1: drop leading zeros within each individual group (a group can never be fully deleted this way, only shortened):

| Group | Original | Leading zeros dropped |
| --- | --- | --- |
| 1 | 2001 | 2001 (no change — no leading zeros) |
| 2 | 0db8 | db8 |
| 3 | 0000 | 0 |
| 4 | 0000 | 0 |
| 5 | 0000 | 0 |
| 6 | ff00 | ff00 (no change) |
| 7 | 0042 | 42 |
| 8 | 8329 | 8329 (no change) |

Result so far: 2001:db8:0:0:0:ff00:42:8329

Step 3 — Compression rule 2: Replace the single longest run of consecutive all-zero groups with :

Groups 3, 4, 5 are consecutive zeros (three in a row) — this is the only zero-run, so it becomes:

Compressed form: 2001:db8::ff00:42:8329

Step 4 — expand back to full notation (reverse Steps 2–3).

- The :: originally stood for 3 zero groups (8 total groups − the 5 groups actually written = 3 missing groups) → put back 0000:0000:0000

- Pad every group back out to exactly 4 hex digits with leading zeros

Expanded form: 2001:0db8:0000:0000:0000:ff00:0042:8329 — matches the original given address.

Step 5 — Determine the network prefix for /64. Each group is 16 bits, so /64 covers exactly the first 4 groups (4 × 16 = 64 bits):

| Portion | Groups | Bits |
| --- | --- | --- |
| Network prefix (/64) | Group 1–4: 2001:0db8:0000:0000 | 64 bits |
| Host/interface ID | Group 5–8: 0000:ff00:0042:8329 | 64 bits |

Network prefix: 2001:0db8:0000:0000::/64 (compressed form: 2001:db8::/64)

Step 6 — Calculate total possible host addresses under this /64 prefix.

Total address length = 128 bits.

Network prefix length = 64 bits.

Remaining host bits = 128 − 64 = 64 bits

Number of possible host addresses = 2⁶⁴. Building this up from known powers of 2:

- 2¹⁰ = 1,024

- 2²⁰ = 1,048,576

- 2⁴⁰ = (2²⁰)² = 1,099,511,627,776

- 2⁶⁴ = 2⁴⁰ × 2²⁴, and 2²⁴ = 16,777,216

2⁶⁴ = 1,099,511,627,776 × 16,777,216 = 18,446,744,073,709,551,616

Total possible host addresses = 2⁶⁴ = 18,446,744,073,709,551,616

5. A university is conducting an online semester examination for 8,000 students simultaneously. At exactly 10:00 AM, the examination server begins transmitting encrypted question papers to all students. The university server is connected through a 10 Gbps high-speed network, while students access the examination portal using different internet connections such as fiber broadband (100 Mbps), home Wi-Fi (20 Mbps), and mobile networks (5–10 Mbps). During the first few minutes of the examination, many students with slower internet connections report incomplete downloads, repeated retransmissions, delayed acknowledgments, and temporary freezing of the examination portal. Network administrators observe that the server continues transmitting data at a high rate even though several student devices are unable to receive and process packets at the same speed. This results in receiver buffer overflow, unnecessary retransmissions, increased network traffic, and delays in delivering the question papers. Analyze the above scenario and identify the primary networking problem. Explain how the mismatch between the sender's transmission speed and the receiver's processing capability affects communication.

SOLUTION:

Primary networking problem identified: a flow-control mismatch between sender and receiver, which escalates into network-wide congestion.

Step 1 — Identify the parameters:

- Sender: exam server, 10 Gbps link, transmitting to 8,000 students simultaneously

- Receivers: highly heterogeneous — fiber broadband (100 Mbps), home Wi-Fi (20 Mbps), mobile (5–10 Mbps)

- Observed symptoms: incomplete downloads, repeated retransmissions, delayed ACKs, portal freezing, receiver buffer overflow, rising network traffic

Step 2 — Diagnose the mechanism: Flow control is the mechanism (e.g. sliding-window protocols) that's supposed to keep a fast sender from transmitting data faster than a receiver can accept and process it. Here that balance breaks down:

- The server transmits at a rate suited to its own 10 Gbps capacity, not tailored to each individual student's much slower link.

- On the slower connections (mobile 5–10 Mbps in particular), incoming data arrives faster than the device/application can drain its receive buffer.

- The receive buffer fills up and overflows, so excess packets are silently dropped.

- Dropped packets are detected (via missing ACKs or timeouts) and retransmitted — but the retransmitted data adds more load onto an already-struggling link.

- This produces a feedback loop: more retransmissions → more traffic → more congestion → more delay → more timeouts → more retransmissions.

- Delayed acknowledgments further slow the effective transfer rate, and the exam portal appears to "freeze" because the client can't keep pace with incoming data.

Step 3 — Why this becomes a network-wide issue: With 8,000 students hitting the server at the same instant, the aggregate volume of retransmissions and buffer-driven packet loss doesn't stay isolated to individual slow connections — it raises overall network traffic, which is classic congestion. So the scenario really has two compounding layers:

- Flow control failure at the individual sender→receiver level (server outpaces a given student's device)

- Congestion at the network level (the sheer aggregate retransmission load degrades performance for everyone, even students on faster links)

Step 4 — How the sender/receiver speed mismatch affects communication:

End-to-end throughput is bounded by the weakest link in the path — here, the student's own connection (as low as 5–10 Mbps), regardless of the server's 10 Gbps capacity. When a sender transmits without regard to the receiver's actual consumption rate, the result is always the same chain: buffer overflow → packet loss → retransmission → wasted bandwidth → increased delay. Effective communication requires the transmission rate to be matched (throttled) to the slowest participant's real receiving capacity, not the sender's theoretical maximum.

Step 5 — Recommended solutions:

- Sliding-window flow control — receiver advertises its available buffer/window size so the sender throttles its rate accordingly

- TCP congestion control (slow start, congestion avoidance, fast retransmit) to react to network-wide load

- Adaptive/rate-based delivery — serve smaller or compressed question-paper payloads to clients detected on low-bandwidth connections

- Load distribution — stagger exam start times, use a CDN, or load-balance across multiple servers so 8,000 simultaneous requests don't all hit one origin server at once

RUBRICS FOR CALCULATION

| Criteria | Weightage | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Clearly interprets problem, identifies all parameters and requirements accurately | Understands problem with minor gaps | Partial understanding; misses key elements | Misinterprets or unable to understand problem |
| Method Selection | 20% | Selects most appropriate method/formula with strong justification | Selects correct method with minor errors | Inappropriate or partially correct method | Unable to select method |
| Solution Process | 25% | Logical, systematic steps with correct approach throughout | Mostly correct steps with minor mistakes | Disorganized steps; multiple errors | No clear process |
| Accuracy of Calculation | 20% | All calculations accurate; correct final answer | Minor calculation errors | Frequent errors; incorrect final answer | No valid calculations |
| Interpretation of Results | 15% | Clearly interprets results with units and engineering relevance | Basic interpretation with minor omissions | Limited or unclear interpretation | No interpretation |